LOCK-OUT SPECS OF PHILIPS HS1

- Semi-automated

- Size: 2.8˝ x 7.4˝ x 8.3˝ (7 cm x 19 cm x 21 cm) D x H x W.

- Weight: With battery and pads case: 3.3 lbs. (1.5 kg)

- Biphasic Truncated Exponential waveform with impedance compensation

- Energy delivery for adult is non-escalating 150 J and 50 J for pedia

- Shock-to-shock cycle time less than 20 seconds

- Provides quick shock in 8 seconds

- With CPR coaching for adult and pedia

- With green On/Off button, blue i-button, orange Shock button

- Stores ECG data for the first 15 minutes and the entire incident’s events and analysis decisions

- ECG data can be downloaded wirelessly via infrared

- Conducts daily, weekly and monthly automatic self-tests

- With blinking green LED light to signify that the device passed its latest self-test

- Tests internal circuitry, waveform delivery system, pads cartridge, and battery capacity, pads integrity test, battery insertion test, upon battery insertion, extensive automatic self-tests and user-interactive test check, device readiness

- Should use pre-connected pads that is compatible both for pedia to adult patients.

- With pediatric attenuator (infant child key) to reduce shock energy during child/infant defibrillation

- Battery:

- Type: 9 Volt DC, 4.2 Ah, composed of disposable long-life lithium manganese dioxide primary cells

- Capacity: Minimum 200 shocks or 4 hours of operating time

- Install-by date: at least 5 years from the date of manufacture

- Standby life: Four years typical when battery is installed

- IP55 rating

- Can withstand 500 lbs crush test

- Carry case should accommodate 1 set of AED pads

- Trainer & training pads available