Technical Specifications:

Brand: Philips

Model Name: PageWriter TC10

ECG Function

- Simultaneous lead acquisition (up to 12 leads)

- Can perform up to 6 channel rhythm strip

- 6 independent events can be marked for later review and analysis

- With five-minute full disclosure of up to 12 leads

- Up to 12 configured Exam Profiles can be specified

- With optional STEMI diagnostic aids (ST Maps, Critical Values, STEMI-CA) to support patient diagnosis

- ECG display is color coded to indicate lead signal quality

- With anatomical lead map that displays the location and label of loose or disconnected leads/electrodes

- Up to 20 lead reversal detection

- Capable of urgent/STAT ECG

Physical features

- Intuitive 1-2-3 operation for fast, efficient workflow

- 7” color touch screen display

- With buttons that light up to guide user

- Fully configurable patient ID fields

- On board “Help” for taking an ECG is available for main ECG functions

- With full screen preview of ECG waveforms to ensure high quality prior to printing

- 6 channel printer with PDF Wi-Fi/USB to external printer

- Printer paper size: 110 mm wide x 28.5 m long roll of thermal-sensitive paper

- Printing speed: 25, 50 mm/sec

- With removable li-on battery

- Battery capacity: 300 typical patient reports or 10 hours continuous operation without printing

- Local archive with maximum of 200 ECGs (PDF/XML file format)

- Transfer ECGs to USB memory stick (PDF/XML file format)

- Dimension: 338 mm x 240 mm x 65 mm

- Weight: 2.5 kg (5.5 lb)

- Capable of optional wireless LAN connection (802.11 a/b/g/n standards)

- Optional barcode reader

- With OPTIONAL five-minute full disclosure of up to 12 leads