Technical Specifications:

Brand: Philips

Model Name: PageWriter TC50

ECG Function

- Simultaneous 12 lead ECG acquisition

- Capable of up to 16 leads by just adding additional leads on the patient interface module

- Can perform up to 16 selected leads rhythm strip

- 10 independent events can be marked for later review and analysis

- With 20-minute full disclosure of up to 12-16 leads

- Up to 12 configured Exam Profiles can be specified

- Limb lead sensitivity of 5, 10, or 20 mm/mV; Precordial lead sensitivity of 2.5, 5, 10, or 20 mm/mV

- Sampling rate of 8,000 samples per second per lead wire

- With optional STEMI diagnostic aids (ST Maps, Critical Values, STEMI-CA) to support patient diagnosis

- ECG display is color coded to indicate lead signal quality

- With anatomical lead map that displays the location and label of loose or disconnected leads/electrodes

- Take ECG from a large touchscreen, keyboard, or patient interface module with a single touch of the green button

- Support pharma stress protocols (timed ECG)

- Full-screen preview of complete ECG report prior to printing

- Up to 20 lead reversal detection

Physical features

- 10.4” color touch screen display

- Intuitive 1-2-3 operation for fast, efficient workflow

- With buttons that light up to guide user

- Fully configurable patient ID fields

- With Trident lead system to reduce tangling and reversals for easier placement and quicker ECG

- On board “Help” for taking an ECG is available for main ECG functions

- Dimension: 3 31 x 40.5 x 20.5 cm (12 x 16 x 8 inches)

- Weight: <8.5 kg (19 lb)

Printer

- With full screen preview of ECG waveforms to ensure high quality prior to printing

- Printing speed: 25, 50 mm/sec

- Paper size: Z-fold letter and A4

Battery

- Removable 2-module lithium-ion battery

- Battery capacity: 30 ECGs on a single charge or 30 minutes of continuous rhythm recording

ECG storage and Network Connectivity

- Performs re-analysis when changing patient data before sending out the ECG report

- Local archive with maximum of 200 ECGs (PDF/XML file format)

- Transfer ECGs to USB memory stick (PDF/XML file format)

- Native-DICOM enabled connectivity with complete and bi-directional communication supports workflow efficiency and lowers the cost of ownership by eliminating 3rd party middleware

- Acquire/print/save/transfer/retrieve ECGs to and from a third party ECG management systems

- Supports LAN and WLAN network connection (802.11 b/g standards)

Other specifications

- Optional barcode reader

- Supports the following lead wire end connectors: alligator clips, wide tab, pediatric clips, welsh bulbs, or snap/tab adaptor