MP-60A Syringe Pump

1	9 infusion modes available: Rate, Time, Bodyweight, drip, sequential, trapezium, loading dose, micro, relay. ★

2	Infusion Rate Range: 0.1-1200ml/h, increment 0.1ml/h

3，	VTBI Setting range 0.1-9999mL.

4，	Infusion accuracy ≤±5%

5,	KVO Rate: 0.1-5.0mL/h

6,	Color TFT touch screen with screen auto lock★

7,	11 level occlusion available★

8,	Anti Free-Flow design when opening the pump door★

9,	Automatic door and Automatic anti-free-flow clamp★

10,	Titration function available★

11,	Calibration function to enable different brand syringe which is compliant to industry standard.

12,	All types of alarm: Near End, Infusion End, Occlusion Alarm, Low Battery, Battery Empty, No Battery, No Power Supply, System trouble, Air Bubble and Reminder Alarm.

13,	high sensitivity air bubble sensor with 7 levels choice★

14,	Strong date storage: 2000 events can be stored★

15,	Connectivity: LAN/WIFI function can connect to Infusion CMS★

16,	Different mounting method, vertical or horizontal.

17	Long time battery support, up to 5 hours with 25ml/h infusion rate.

18	European CE or US FDA certification★

19	 Very light design, only 1.2Kg with battery. ★

20	Barcode Scanner for quick patient information input★

21, 	1.1	All set should be European CE/US FDA approved

22	Drop sensor specific mounting position should be available

23	Drug Library 1000 types★