Goldway G40e

- 12.1-inch color TFT LED display

- 800 X 600 pixels resolution

- Maximum display of 11 channels; auto channel height adjustment function

- Lightweight & ergonomic design

- Parameters: 3/5 lead ECG, HR, RESP, NBP, SpO2, PR, dual TEMP

- Optional dual IBP, etCO2 monitoring, and Cardiac Output function

- With ST segment analysis and 21 calls of arrhythmia

- FAST SpO2 technology with anti-motion and low perfusion performance

- Navigation wheel for information input and screen navigation

- Backlit function buttons for quick access to key functions

- Bed-to-bed overview function for up to 32 monitors without central monitoring system

- Bi-directional communication via LAN networking to a central monitoring system

- Standard HL7 output

- Fan-less design to reduce dust deposits

- Applications from neonate to adult

- With OxyCRG & minitrend

- With big number screen, dynamic trend screen, & OCRG screen

- 4 user-defined layout

- Timing clock configurable from 5-30mins

- 1,600 sets NBP table

- 1,200-hour trend graph of all parameters (with data manager function)

- 120-minute full disclosure ECG 1 waveform review (with data manager function)

- 200 sets alarm events & 200 sets arrhythmia events

- With drug calculation

- With HR variation & dynamic blood pressure analysis

- Rechargeable lithium ion battery with 2.5 hrs of operation

- Optional built-in 3-channel recorder

- Standard external interface available: ECG analog output, VGA output, Nurse call, LAN, USB

- Patient data export using a USB data manager

- Net weight: approximately 4.2kg